# Fluxonium Reset Project Handoff

Context for continuing the physics audit coding work and reinforcement learning discussion

# How to use this handoff

This document captures the current project model, decisions confirmed with Dr. Fu, completed audit results, the present coding checkpoint, and questions for the reinforcement learning paper. Attach it to a new chat and ask to continue from the Current checkpoint section.

# Project objective

The project optimizes an active reset protocol for a fluxonium qubit coupled to a lossy microwave cavity. The current numerical target is final ground-state population at or above 0.99 while reducing total reset time. The existing baseline uses CMA-ES. The project version can optimize flux and charge-drive controls together; the professor's reference code optimizes only the flux pulse and keeps the charge drive fixed.

# Physical reset pathway

The intended route is |e,0> to |f,0> by a microwave charge drive, followed by coherent transfer from |f,0> to |g,1> through the fluxonium-cavity coupling. Cavity decay then removes the photon and leaves |g,0>.

Half flux, Phi_ext/Phi_0 = 0.5 and external phase pi, is the symmetric sweet spot and the idle endpoint.

Near quarter flux, Phi_ext/Phi_0 = 0.25 and external phase pi/2, parity is broken and the g-to-f charge matrix element becomes nonzero.

The quarter-flux region is a calibrated interaction neighborhood, not a fundamental requirement that every optimized pulse must hit exactly 0.25.

The optimized sine basis forces the flux pulse to begin and end at pi, while permitting a flexible path between the endpoints.

# Basis conventions

Fluxonium matrices are stored in a fixed truncated LC or harmonic-oscillator basis. This is a computational coordinate system, not a claim that fluxonium is harmonic. At a chosen flux, diagonalizing the full fluxonium Hamiltonian produces the physical energy eigenstates |g>, |e>, |f>, and higher states as vectors in the same LC basis.

n_operator with energy_esys false returns the charge operator in the fixed LC basis.

n_operator with energy_esys true returns the same operator transformed into the energy eigenbasis at that object's flux bias.

The full simulator basis is fluxonium LC basis tensor cavity Fock basis. With fluxonium cutoff 20 and cavity states |0>, |1>, the combined dimension is 40.

A fixed LC basis is convenient because the flux-dependent Hamiltonian evolves in time. An instantaneous energy basis would itself move and require an additional basis-motion term.

# Confirmed experimental and modeling choices

| Choice | Confirmed meaning |
| --- | --- |
| Initial state | A 50 percent ¦g,0> and 50 percent ¦e,0> mixture is the most common practical case. |
| Cavity cutoff | At most one cavity photon aligns with the experiment, motivating the ¦0>, ¦1> truncation. |
| Fidelity target | Use final ground-state population 0.99 for the current study. |
| Stored frequencies and matrix elements | omega_ef, omega_gf, n_ef, n_gf, g, and related values come from experimental measurements or calibration. |
| Control architecture | Professor code optimizes the flux pulse only. Project code can optimize flux and charge-drive coefficients jointly. |

# Completed physics audit results

| Audit result | Finding |
| --- | --- |
| Half-flux spectrum | g-to-e = 0.697075 GHz; e-to-f = 4.323951 GHz in the audited static model. |
| Quarter-flux matrix elements | Computed ¦<e¦n¦f>¦ = 0.3044204 and ¦<g¦n¦f>¦ = 0.3748523, matching stored 0.304437775 and 0.374859134 closely. |
| Parity check | At half flux, ¦<g¦n¦f>¦ is approximately zero because ¦g> and ¦f> have the same parity and n is odd. |
| Quarter-flux bare detuning | At 0.25 Phi_0, f_gf = 6.548445 GHz and cavity frequency = 6.517217 GHz, for about 31.23 MHz detuning. |
| Exact bare resonance | With cutoff 25, f_gf equals the configured cavity frequency at Phi_ext/Phi_0 = 0.253980967. |

# Meaning of the resonance calculation

The bare resonance condition compares |f,0> with |g,1>. Without coupling, their energy curves cross when the fluxonium g-to-f transition equals the cavity frequency. Coupling adds an off-diagonal matrix element proportional to g times <g|n|f>. The coupled eigenstates are mixtures of |f,0> and |g,1>, often labeled psi minus and psi plus, and their eigenvalues form lower and upper dressed branches.

A dressed branch is an eigenvalue of the full coupled Hamiltonian tracked as flux changes. The subspace weight |<f,0|psi>| squared plus |<g,1|psi>| squared identifies which full-system eigenstates belong mainly to the reset pair. The avoided-crossing gap shows that coherent transfer between |f,0> and |g,1> is available.

The bare-state average energy does not reveal the coupling because the interaction is primarily off diagonal. The full-system eigenvalue splitting reveals it. The static avoided-crossing plot validates the coherent doorway; it does not yet demonstrate irreversible reset.

# Why cavity decay completes reset

Without loss, the coupled Hamiltonian permits reversible oscillation between |f,0> and |g,1>. The cavity collapse operator is proportional to the photon annihilation operator. It maps |g,1> to |g,0> and annihilates |f,0>, which already has zero photons. After emission, the excitation resides in the external environment and cannot coherently reconstruct |f,0>.

# Current checkpoint

The next audit is a time-domain population simulation at the resonant flux. The first checkpoint is complete: construct a fluxonium object at Phi_ext/Phi_0 = 0.253980967, obtain its low-energy eigenstates, and obtain n in the LC basis.

Continue interactively in these checkpoints:

Construct the tensor-product states |f,0>, |g,1>, and |g,0> in the combined LC by cavity basis.

Construct the static coupled Hamiltonian H_flux tensor I plus I tensor H_cavity plus g n tensor sigma_y.

Create projectors onto |f,0>, |g,1>, and |g,0>.

First evolve without cavity decay and confirm reversible |f,0> to |g,1> oscillations.

Then add the cavity collapse operator and confirm that population accumulates in |g,0>.

Plot P_f0(t), P_g1(t), and P_g0(t), and verify that their behavior matches the reset story.

# Questions still open in the audit

Resolve the exact cycles-per-nanosecond versus radians-per-nanosecond convention used by mesolve and every stored frequency.

Run cutoff convergence for relevant gaps, matrix elements, and final fidelity at N equal to 20, 25, and 30.

Establish a reproducible professor-code CMA-ES baseline with seed, runtime, evaluation count, best fidelity, and saved pulse.

Profile one objective evaluation and identify time spent in Hamiltonian construction and mesolve.

Document whether the optimized pulse reaches exact bare resonance, passes through it, or uses a nearby dressed optimum.

Validate the one-photon cavity truncation dynamically by confirming that the experimental model does not require higher photon states.

# Reinforcement learning paper context

Source: References/Research-Texts/RL-Quantum.pdf. Chapters 1 through 4 have received a light first read. The paper studies deep reinforcement learning for superconducting-qubit readout pulse shaping, rather than this exact fluxonium reset task.

## Bloch equation observation

The paper reports that generic QuTiP master-equation simulation was too slow for its training workload and derives cavity Bloch equations for selected expectation values such as the cavity amplitude and photon count. This is not automatically a drop-in replacement for the current mesolve model. The paper's equations close because its dispersive readout Hamiltonian has a specific structure.

For this project, mesolve remains the trusted reference until a reduced model reproduces the relevant population trajectories and final fidelity within a declared tolerance. The audit question is whether the reset dynamics can be represented by a closed set of populations and coherences instead of the full density matrix.

## Tentative reinforcement learning formulation

| Component | Candidate definition for this project |
| --- | --- |
| Environment | The fluxonium and cavity simulator, including cavity decay and control limits. |
| Observation | Time plus selected populations or coherences, device parameters, or another compact state representation. |
| Action | Flux and charge-drive amplitudes for the next segment, or coefficients describing the entire pulse. |
| Reward | Final P_g0, with possible penalties for duration, amplitude, bandwidth, or roughness. |
| Policy | A neural network that chooses sequential control actions or predicts an entire pulse. |

## Questions for the next paper reading

What exactly does the agent observe at each step?

What constitutes one action and one episode?

Is the pulse generated sequentially or in one policy pass?

How sparse or dense is the reward, and how is pulse duration rewarded?

How many simulator evaluations does training require?

What optimizers and hand-designed pulses are used as baselines?

Does the learned policy generalize across device parameters, initial states, noise, or target durations?

Does reinforcement learning outperform the baseline under equal wall-clock time or equal simulator-evaluation budgets?

## CMA ES and reinforcement learning comparison plan

Use the same simulator, control bounds, control parameterization, fidelity target, and compute budget for every optimizer. Report success probability for reaching 0.99, evaluations to threshold, wall-clock time, final fidelity, and robustness under parameter perturbations. Reinforcement learning is most compelling if one trained policy transfers across a family of device conditions or supports feedback. CMA-ES may remain preferable for one fixed deterministic open-loop pulse.

# Important project files

| Path | Role |
| --- | --- |
| params.json | Current flat mixture of device, protocol, and numerical parameters. |
| source/simulator.py | Current project simulator with joint flux and charge-drive controls. |
| References/Code/cmaes_autotune 1.py | Professor reference CMA-ES implementation. |
| playzone/fluxonium_model_audit.py | Planned Marimo physics audit. |
| References/Research-Texts/RL-Quantum.pdf | Reinforcement learning and qubit-readout thesis. |

# Recommended next message

Continue the time-domain population audit from checkpoint 2. Guide me interactively rather than writing the full code. I have already constructed fluxonium at the resonant flux, its low-energy eigenstates, and n in the LC basis.